BID SPECIFICATION SHEET FOR FB19-032

# ACCESS LOCKS ADMINISTRATION CORRIDOR POPPLEWELL HALL

# PART 1 – GENERAL

1.1	SECTION INCLUDES

- Provide, install, commission, and warranty an On-line Stand Access Control System (ACS).

- The ACS shall interface with Owner supplied PC, router/switch, and LAN infrastructure.

1.2	QUALITY ASSURANCE

- Provide documentation verifying the utilization of authorized factory trained software installation personnel.  Documentation shall include certificates of completion for access control software installation training.  Only factory direct training certificates will be accepted.

- Mandatory Pre-Bid Conference will be held on Wednesday, August 15, 2018 at 10:00AM in Popplewell Hall room 206.

- Installation Conference.  The successful bidder shall attend all conferences scheduled by the owner during the installation to coordinate and collaborate will all other effected trades and services.

- Coordinate all LAN/IP requirements with owner representative.

1.3	SUBMITTALS

- Submit shop drawings, riser diagrams, field-wiring diagrams, elevation drawings indicating the location of all ACS components including the electronics at each opening and the location/address of the Owner provided PC.

- Submit catalog cuts showing technical data necessary to evaluate the equipment and other descriptive data necessary to describe fully the equipment proposed.

- Submit factory training certifications for all personnel that will be installing the ACS and electronic locking devices

- Submit the Pricing Page 4, included in this document completely and accurately.

PART 2 – PRODUCTS

2.1	MANUFACTURERS

A.	Acceptable ACS Software:

1.  S2 Security

2.  RS2 Technology

3.  Avigilon

4.  AccessNSite

5.  Owner approved equal

B.	Acceptable Integrated ACS Electronics

1.  Schlage LEMD Mortise Deadbolt – no substitution

2.  Schlage GWE Bluetooth Gateway IP/POE or RS485 – no substitution

2.2	MATERIALS

- ACS Software

  - Minimum of 5 user license.  Assign all warranties and software license rights to the Owner representative.

- ACS Integrated Card Readers Openings

  - Provide multi-technology 125 KHz and 13.56 MHz readers.

  - Provide concealed door status monitor

  - Provide integrated request to exit switch

  - Provide “AA” standard batteries and battery case

  - Provide ACS IP/POE or RS485 control modules.

- Owner shall provide cylinders and keying.  Order appropriate lever and rose. System Design: Schlage  LEMD - ADD

- ACS Programming device

  - Provide Allegion Engage commissioning application for Droid and iPhone.

- ACS Software

  - Provide ACS Software

    - System Design:  S2 Web based ACS

PART 3 – EXECUTION

  - EXAMINATION

Provide and install 5 (five) integrated locking systems in the Executive Suites area and 1 (one) integrated locking system on the 1st Floor HR Office.  Remove existing mortise lock, if required enlarge the mortise pocket in the openings.  Provide owner representative all parts and pieces of existing locks once removed from the opening.

Install 2 (two) Gateway IP/POE or RS485 Bluetooth transceivers in the Executive Suite area and 1 (one) on the 1st floor...  Gateways to be within 30’ of LE lock location and will be terminated in IT closet.  Cable (either Cat5/6 or RS485) shall penetrate 1 fire wall and shall have fire caulk per code.

Install software on Owner provided PC, switch/router, and LAN environment.   Provide a minimum of 4 (Four) hours of software training for up to 4 (four) owner representatives to include but not limited to:

Software Registration and Activation

Enter Smart Time Zones per owner definition

Enter Door Names, Auto Unlock, and Holiday Schedules

Import User First Name and Last Name using CSV format owner supplied file

Enroll existing owner credentials (minimum of 5)

Assign Access Rights

Test credentials (minimum of 3), run audit trail and run Audit Reports

Coordinate installation of access control system with all Owner groups.

BID INFORMATION

Sealed bids must be submitted to the Purchasing Office by 2:00 p.m. central time on August 29, 2018 in Popplewell Hall, Room 221, 4525 Downs Drive, St. Joseph, MO 64507.  Electronic or faxed bids will not be accepted.  All envelopes must be clearly marked with FB19-032. All questions should be directed to Purchasing Manager, Kelly Sloan, (816) 271-4465, purchase@missouriwestern.edu

Missouri Western reserves the right to accept or reject any or all items of this bid.

Include with your bid:

- Current signed W-9

- Return pricing sheet found on page 4

- Include any addendum(s) with your initials

- Completed Invitation to Bid (separate document)

PRICING SHEET FOR FB19-032

# ACCESS LOCKS ADMINISTRATION CORRIDOR POPPLEWELL HALL

- ACS Pricing

Equipment Pricing (in US Dollars):$

Software Installation Labor: 	$

Project Total: 	$

- ACS Option Pricing

LEMD				$___________________

Annual Service Agreement Yr 2	$

Annual Service Agreement Yr 3	$

Annual Service Agreement Yr 4	$

Annual Service Agreement Yr 5	$

Time and Materials Labor Rate	$

Contractor shall provide Missouri Western State University with a turnkey solution, which meets or exceeds the requirements listed in this Formal Bid.  The Pricing submitted shall be good for a period of up to six months from the time of submittal.  Submitting a proposal is no guarantee of a contract or agreement with Missouri Western State University.  A separate contract will be issued upon award of the Purchase Order.

Authorized Signature 								Date

Company ____________________________________________