FORMAL BID FB24-004

FOR

SOFTBALL INFIELD AT SPRING SPORTS COMPLEX

Submission Deadline:  JUNE 16, 2023 2:00 p.m. Central Time

Questions and/or requests for clarification of this FB should be submitted via email to the Purchasing Manager, Kelly Sloan at purchase@missouriwestern.edu. All questions and/or clarifications can be sent at any time regarding this FB to the Purchasing Department until 12:00pm on June 9, 2023.  Questions received after this date may not be answered.  Please reference FB24-004 on all correspondence.   Answers to the submitted questions will be e-mailed to each vendor as an addendum.  It is the responsibility of all interested parties to read this information and return it as part of the FB confirming receipt.

BID SPECIFICATION SHEET FOR FB24-004

SOFTBALL INFIELD AT SPRING SPORTS COMPLEX

SCOPE

The elevation and level of the infield (skinned) playing surface at Missouri Western State University (MWSU) softball field needs to be addressed.  Specific areas of the scope of work required are listed below.  The work along the foul territory warning track in both right/left field will be alternate to the priority work on the infield.

INFIELD RENOVATION WORK

- Remove padding from backstop wall and mark all bases, plates and irrigation heads

- Import 65 tons of DuraEdge Collegiate Infield Mix

- Existing surface will need tilled to at least a depth of 1”

- Regrade skinned infield surface to create a uniform sub-grade

- Add DuraEdge material to infield and laser grade to help prevent movement of infield material following installation

- Water and roll infield to DuraEdge specifications

- Provide and install 2 tons of Turface Pro League Red

- Install new base wells, home plate and pitchers plate level at infield elevation

- Adjustment of irrigation heads to ¼” below final grade

- Clean grass edge on the infield arc

- Furnish and add sod as needed at the corners of the arc to assist with the adjustment of the infield elevation

WARNING TRACK WORK (ALTERNATE)

- Clean edge of grass in foul territory down both the right/left field lines

- Laser grade existing warning track (foul territory)

- Import 1 truckload (22-24 tons) of Redshale or crushed Red Lava and install

- Laser grade and compact new material

MANDATORY PRE-BID MEETING

No meeting will be held.

BID INFORMATION

Sealed bids must be submitted no later than June 16, 2023 2:00 p.m. Central Time to:

Missouri Western State University

Purchasing, Popplewell Hall 221

4525 Downs Drive

St. Joseph, MO  64507

Electronic or faxed bids will not be accepted. MWSU reserves the right to accept or reject any or all bids. Responses must be signed and sealed in mailing envelopes with the Vendor’s name and address clearly written on the outside.  The Formal Bid Number (FB24-004) shall also be clearly marked on the outside of the envelope or package.  Proposals may not be accepted if the FB number is not on the outside of the envelope.

MWSU reserves the right to award to the bidder whose bid complies with all mandatory specifications and requirements and is the lowest and best bid for supplies. MWSU reserves the right to accept or reject any or all items of this bid.

All questions should be directed to Purchasing Manager, Kelly Sloan, (816) 271-4465, purchase@missouriwestern.edu

Include with your bid:

- Current signed and dated W-9

- Pricing sheet on page 4

- Include any addendum(s) with your initials

- Invitation for Bid (separate page with red font)

PRICING SHEET FOR FB24-004

SOFTBALL INFIELD AT SPRING SPORTS COMPLEX

Infield Renovation Work:

$________________

Warning Track Work (alternate):

$________________

Company Name: _________________________________________________

Printed Name: __________________________________________________

Authorized Vendor Signature:

Phone Number: ________________________________________________